- CATIA V5

- LaunchPad

- V 1.7.0

- Michael Neukirchen

mailto:neukirch@de.ibm.com

- Content

1	Preface	3

2	What’s new	3

2.1	1.0	3

2.2	1.1	3

2.3	1.2	3

2.4	1.4	3

3	Installation	4

3.1	Windows	4

3.2	Unix	4

3.3	Configuration	4

4	How does it work?	5

4.1	In general	5

5	 Features - Functions	6

5.1	First start, Configuration	6

5.1.1	CatiaStarter.cfg	6

5.1.2	CLPGeneral.tcl	7

5.2	Integration with ST or other PDM	7

5.2.1	ST configuration: Tool definition	8

5.2.2	ST configuration: Apply a script to the Edit Hook	8

5.2.3	ST configuration: Add a user defined command	9

5.2.4	ST configuration: Edit the data model	9

6	Launch the program	11

7	Copyright Notice	12

8	Appendix: Environments / compatibilities– not implemented yet (feedback needed!)	14

9	Displaying of MOTDs	15

9.1	Tags Supported by the Library	15

9.2	Tags Supported by the Library that are not in the spec	16

9.3	Unsupported tags	16

9.4	Notes	17

9.5	Exceptions	17

Pictures

Picture 1: ST tool definition	8

Picture 2: Add the script to the hook	9

Picture 3: Choosing the entry point for the script	9

Picture 4: Data Model update	10

Picture 5: Profile Card	10

Picture 6: LaunchPad Panel	11

Picture 7: Small Launch Pad	11

# Preface

I have got several requests for a launching tool for CATIA V5.

This version is a very simple tool. The user can just select the CATIA he wants to launch and it will be started. The different versions are driven by the configuration data set.

You can install the programm on a server – the configuration file has to reflect the file names as seen from the client (either mounted directories or mapped drives).

If you have additional requests, drop me a mail – I am very interested to get some feedback.

# What’s new

## 1.0

Initial release.

## 1.1

Tests done for AIX.

## 1.2

Name Switch to CatiaLaunchPad.

Integration for ST added.

## 1.4

Bug Fixes.

Selection of programs via list box.

Support for icons/logos

Support of Message of the Day (MOTD)

## 1.6

Bug Fixes.

Message, if program could not be started

Multi-Column structuring

## 1.7

Bug Fixes: In some case a program did not start – probably due to missing stdout

Debug mode added

Panel will iconify until a program has finished.

# Installation

## Windows

- Unzip the file and place everything in one directory.

## Unix

- Get the tclkit for your operating system at

  - http://www.equi4.com/pub/tk/8.4.1

- Build a startup script: tclkit CatiaStarter.kit $1

- Tested on AIX (Version 1.1)

## Configuration

- See the chapter below, to add your individual configuration

# How does it work?

## In general

The macro is written in TCL/TK. It launches the program using a list of predefined items.

These items represent the different CATIA environments or modes to start CATIA (e.g. admin mode). The starting mode is stored centrally (on Windows using the registry) so that other programs can access the current CATIA environment.

# Features - Functions

## First start, Configuration

Edit the config data set CatiaStarter.cfg in the config directory or if needed the CLPGeneral.tcl data set.

### CatiaStarter.cfg

Here you can define the commands used to start CATIA with different environments.

####################################################################

# the catia starter definition

####################################################################

#

# each command has 4 entries

#

# 1st: Name the user can select in brackets []

# 2nd: The name of the command to be invoked (Command= at the beginning of the line)

# 3rd: Additional parameters such as environment definitions ... (Options= at the beginning of the line)

# 4th: Additional information, which will be placed to be picked up by a pdm system

# 5 th: An Icon which can be placed in front of the Name

#

#

[V5R12 SP3 Admin]

Command=C:/Programme/Dassault Systemes/B12/intel_a/code/bin/CNEXT.exe

Options=-admin -env CATIA.V5R12.B12 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R12,SP3,HF5

#Gif=E:\$user\src\catia\CatiaStarter\CatiaLaunchPad v1.4\logos\audi.gif

[V5R12 SP3]

Command=C:\Programme\Dassault Systemes\B12\intel_a\code\bin\CATSTART.exe

Options=-run "CNEXT.exe" -env CATIA.V5R12.B12 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R12,SP3,HF5

[V5R10 SP4]

Command=C:/Programme/Dassault Systemes/B12/intel_a/code/bin/CNEXT.exe

Options=-env CATIA.V5R10.B10 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R10,SP4

### CatiaStarter.cfg with structured commands

Starting with 1.5.0 the CLP provides the possibility to group the different start commands. Simply inster a title tag in the config file and you will see different columns in the selection area.

####################################################################

# the catia starter definition

####################################################################

#

# each command has 4 entries

#

# 1st: Name the user can select in brackets []

# 2nd: The name of the command to be invoked (Command= at the beginning of the line)

# 3rd: Additional parameters such as environment definitions ... (Options= at the beginning of the line)

# 4th: Additional information, which will be placed to be picked up by a pdm system

# 5 th: An Icon which can be placed in front of the Name

#

#

<V5R12>

[V5R12 SP3 Admin]

Command=C:/Programme/Dassault Systemes/B12/intel_a/code/bin/CNEXT.exe

Options=-admin -env CATIA.V5R12.B12 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R12,SP3,HF5

#Gif=E:\$user\src\catia\CatiaStarter\CatiaLaunchPad v1.4\logos\audi.gif

[V5R12 SP3]

Command=C:\Programme\Dassault Systemes\B12\intel_a\code\bin\CATSTART.exe

Options=-run "CNEXT.exe" -env CATIA.V5R12.B12 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R12,SP3,HF5

<V5R10>

[V5R10 SP4]

Command=C:/Programme/Dassault Systemes/B12/intel_a/code/bin/CNEXT.exe

Options=-env CATIA.V5R10.B10 -direnv "C:/DOKUME~1/ALLUSE~1/Anwendungsdaten/DassaultSystemes/CATEnv"

AddInfo=R10,SP4

### CLPGeneral.tcl

In this data set, you can specify general behaviours of the Launch Pad.

####################################
# general config for the launch Pad
####################################

#
#  persistent env info (registry, or home directory)
#
set usePersistentEnv true

set height 5

set MOTD "E:/\$user/src/catia/CatiaStarter/CatiaLaunchPad v1.4/MOTD"

set motdHeight 10

set motdWidth 70

set widthSelection 70

set heightSelection 5

# same width MOTD and selection area

set motdWidth $widthSelection

# turn the debug mode on

# set debug “yes”

#### usePersistentEnv

true: the Environment will be stored in a data set and can be retrieved by a PDM System (see below)

#### height

The maximum number of programs to start, which can be seen. If the number of program exceeds that limit, a scrollbar will be attached.

#### Message of the Day

If you set the variable MOTD, then the program will sho a message of the day. The path for the folder of the message(s) has to be specfied. The Launch Pad will call the data set index.html and display that.

The Launch Pad can display basic features of html pages (see Appendix).

motdHeight and motdWidth specify the size of the MOTD area.

If you comment the line – no MOTD will be shown.

## Integration with ST or other PDM

PDM tools normally allow only one application to edit a specific type of data. So the necessary additional information regarding the environment has to be stored somewhere.

The LaunchPad will put the currently used environment in the Home Directory of the user. SMARTEAM can pick up this info later to get the necessary information. Persistent env info is necessary if you want to use this feature.

If an environment is set previously (e.g. by SMARTEAM) this environment will be chosen for the default, i.e. this environment is shown in the drop down control. If no environment is given or the environment set is not in the configuration of the LaunchPad, the first environment defined will be used.

If you want to let the LaunchPad work together with other PDM Systems, the communication is pretty simple:

Get the current environment

You can retrieve the current environment from the registry through:

%HOMEDRIVE%\%HOMEPATH%\CATIALaunchPad.txt

$HOME/ CATIALaunchPad.txt

Store that info together with the CATIA files in the PDM.

Call CATIA with a given environment

And if you want to start CATIA with a given file and environment, first set the environment in the file and then call the LaunchPad with the CATIA filename as the first parameter.

### ST configuration: Tool definition

In order to launch CATIA with the right environment, you have to specify the LaunchPad as the editing tool for CATIA files. The LaunchPad will suggest the given environment and then start CATIA.

Picture 1: ST tool definition

### ST configuration: Apply a script to the Edit Hook

Before the LaunchPad is started, the environment is picked up from the profile card of the selected CATIA file and written to the home directory. To achieve this, the before edit hook is used.

Use the script maintenance to attach the script mn_env2reg.bs to the hook.

Picture 2: Add the script to the hook

Picture 3: Choosing the entry point for the script

### ST configuration: Add a user defined command

A script is provided, which enters on all items below the selected item the current CATIA environment. This script has to be added through the script maintenance and menu editor.

### ST configuration: Edit the data model

The program stores the environment in a special field: CATENV. This field has to be introduced for all CATIA file types.

Picture 4: Data Model update

Edit the profile cards accordingly, to enable the user to see the environment.

Picture 5: Profile Card

# Launch the program

After launching the LaunchPad, the user can select the desired CATIA in the drop down combo box. Selection of the ok button will start CATIA, Selection of Cancel will abort the program.

Picture 6: LaunchPad Panel

If the LaunchPad is called with a CATIA file name as argument, CATIA will be opened with the given file. The filename is shown in the panel. If no CATIA file is given – the message will be shown as above.

A ‘small’ version without logos and MOTD will look like this:

Picture 7: Small Launch Pad

# Copyright Notice

Copyright (c) 2007 Michael Neukirchen

The following terms apply to all files associated with the software unless explicitly disclaimed in individual files.

The authors hereby grant permission to use, copy, modify, distribute, and license this software and its documentation for any purpose, provided that existing copyright notices are retained in all copies and that this notice is included verbatim in any distributions. No written agreement, license, or royalty fee is required for any of the authorized uses. Modifications to this software may be copyrighted by their authors and need not follow the licensing terms described here, provided that the new terms are clearly indicated on the first page of each file where they apply.

IN NO EVENT SHALL THE AUTHORS OR DISTRIBUTORS BE LIABLE TO ANY PARTY FOR DIRECT, INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OF THIS SOFTWARE, ITS DOCUMENTATION, OR ANY DERIVATIVES THEREOF, EVEN IF THE AUTHORS HAVE BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

THE AUTHORS AND DISTRIBUTORS SPECIFICALLY DISCLAIM ANY WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, AND NON-INFRINGEMENT.  THIS SOFTWARE  IS PROVIDED ON AN "AS IS" BASIS, AND THE AUTHORS AND DISTRIBUTORS HAVE NO OBLIGATION TO PROVIDE MAINTENANCE, SUPPORT, UPDATES, ENHANCEMENTS, OR MODIFICATIONS.

GOVERNMENT USE: If you are acquiring this software on behalf of the U.S. government, the Government shall have only "Restricted Rights" in the software and related documentation as defined in the Federal  Acquisition Regulations (FARs) in Clause 52.227.19 (c) (2).  If you are acquiring the software on behalf of the Department of Defense, the software shall be classified as "Commercial Computer Software" and the Government shall have only "Restricted Rights" as defined in Clause 252.227-7013 (c) (1) of DFARs.  Notwithstanding the foregoing, the authors grant the U.S. Government and others acting in its behalf permission to use and distribute the software in accordance with the terms specified in this license.

The script uses the tcl html Library with the following license agreement:

Sun Microsystems, Inc.  The following terms apply to all files

associated with the software unless explicitly disclaimed in individual

files.

The authors hereby grant permission to use, copy, modify, distribute,

and license this software and its documentation for any purpose, provided

that existing copyright notices are retained in all copies and that this

notice is included verbatim in any distributions. No written agreement,

license, or royalty fee is required for any of the authorized uses.

Modifications to this software may be copyrighted by their authors

and need not follow the licensing terms described here, provided that

the new terms are clearly indicated on the first page of each file where

they apply.

IN NO EVENT SHALL THE AUTHORS OR DISTRIBUTORS BE LIABLE TO ANY PARTY

FOR DIRECT, INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES

ARISING OUT OF THE USE OF THIS SOFTWARE, ITS DOCUMENTATION, OR ANY

DERIVATIVES THEREOF, EVEN IF THE AUTHORS HAVE BEEN ADVISED OF THE

POSSIBILITY OF SUCH DAMAGE.

THE AUTHORS AND DISTRIBUTORS SPECIFICALLY DISCLAIM ANY WARRANTIES,

INCLUDING, BUT NOT LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY,

FITNESS FOR A PARTICULAR PURPOSE, AND NON-INFRINGEMENT.  THIS SOFTWARE

IS PROVIDED ON AN "AS IS" BASIS, AND THE AUTHORS AND DISTRIBUTORS HAVE

NO OBLIGATION TO PROVIDE MAINTENANCE, SUPPORT, UPDATES, ENHANCEMENTS, OR

MODIFICATIONS.

RESTRICTED RIGHTS: Use, duplication or disclosure by the government

is subject to the restrictions as set forth in subparagraph (c) (1) (ii)

of the Rights in Technical Data and Computer Software Clause as DFARS

252.227-7013 and FAR 52.227-19.

# Appendix: Environments / compatibilities– not implemented yet (feedback needed!)

I am just thinking what will happen, if we use environments for a longer time. Your feedback is appreciated – drop me a mail!!

We face the problem that environments are evolving over the time. So the following classification will be used:

    - General compatible environment

    - Friendly environments

    - Incompatible environments

To illustrate that, let’s take the case of a company which is a supplier for 2 OEMs say OEM1, OEM2 and which has an internal environment SUPP.

The OEMs may have the environment

    - OEM1_2004

    - OEM2_2004

Over the time, OEM1 switches to OEM1_2005 and OEM2 to OEM2_2005. The internal environments have been SUPP_2004_1, SUPP_2004_2, SUPP_2005_1.

Lets take a view into the future, in 2005 and take a definition as follows:

GCE – General compatible environment

Set GCE SUPP_2005_1

Friendly environments

Set friendly(OEM1_2005) “OEM1_*”

Set friendly(OEM2_2005) “OEM2_*”

Consider the following use cases:

Use Case 1: A CATIA file is launched from ST

Current environment is the GCE

    - each file will be called

Current environment not GCE

    - if environment is not in the friendly list, a warning will be issued

Use Case 2: Add the current environment to a set of CATIA files

Environment is the GCE

    - Add the current GCE to environment attributes which belong to GCEs or which are empty

Environment is other (e.g. OEM1_2004)

the entries other than GCEs are set to the current environment (OEM1_2004)

# Displaying of MOTDs

## Tags Supported by the Library

title, /title

The document title. No markup is permitted between the opening and closing tags, otherwise the title string is truncated at the first non /title tag.

h1, h2, h3, h4, h5, h6 /h1, /h2, /h3, /h4, /h5, /h6

Heading levels.

p, /p (optional)

Paragraphs are rendered block left, with one blank line.

pre, /pre

Preformatted text. All whitespace is preserved. Any markup is permitted inside of a pre tag. Rendered in a mono-spaced font.

address, /address

Address delimeters, rendered in an italic font.

blockquote, /blockquote

Block quotes nest on the left with the list constructs and other blockquote tags, but the right-margin indent level does not nest. They are rendered in italics, with a blank line above and below.

ul li... /ul

Unordered lists. Rendered with a different marker symbol for each indent level. The default symbols are: O*=+-o\xd7\xb0:\xb7

ol li... /ol

Ordered lists. Rendered with a number for each list element. Each indent level has its own numbering.

dir li... /dir

Directory listings. Rendered the same as unordered lists.

menu li... /menu

Menu lists. Rendered with a unique (->) list element symbol.

dl (dt...,dd)... /dl

Definition lists. There may be multiple dt elements per dd element.

cite

Citation. Rendered in italics.

code

Code fragment. Rendered in a mono-spaced font.

em

Emphasized. Rendred in italics.

kbd

Keyboard or user input. Rendered in a bold, mono-spaced font.

samp

Sample. endered in a mono-spaced font.

strong

Different from em. Rendered in bold face.

var

A variable reference. Rendered in italics.

b

Bold face.

i

Italics

tt

A typewriter, or mono-spaced font.

br

A line break.

hr

A horizontal rule. Rendered as a full width horizontal line.

a

A hyperlink anchor. The supported parameters are name and href. The name parameter is the target, or destination of a hyperlink. It can be referenced by using the HMgoto callback, with name as an argument.

The href tag is accessed by two callback routines. HMlink_setup is called once for each hyperlink. It is expected to associate any application specific behavior with a hyper-link. The default version of HMlink_setup arranges for hyperlinks to be highlighted/dehilighted upon cursor entry and exit. When the link is activated (with mouse button 1) HMlink_callback, which is supplied by the application, is called.

Any additional parameters supplied with the a tag are passed as an argument to the call-back routines.

img

An in-line image. Inline images are obtained in two steps. First, The align, alt, src, and ismap attributes are handled by the library.

As each inline image tag is seen, the procedure (supplied by the application) HMset_image handle src is called. The application is expected to locate the image (using src, and create a TK image with it.

The application then calls HM_got_image handle image, where handle was the value passed to HM_set_image, and image is either the name of a TK image, or an error message.

If the parameter ismap was supplied, then a call to HMlink_callback is made when the image is clicked on.

## Tags Supported by the Library that are not in the spec

u

underline

strike

Text is rendered with a horizontal line through it.

center

All text and images starting at the next line break are centered.

dfn

Definition terms

## Unsupported tags

head /head

This tag is used by servers, and has no meaning in the context of document display.

body /body

This tag is used by servers, and has no meaning in the context of document display.

meta /meta

This tag is used by servers, and has no meaning in the context of document display.

base /base

This tag does not effect the document display. The functions HMtag_base and HMtag_/base need to be supplied by the application.

link

This tag does not effect the document display. The function HMtag_link needs to be supplied by the application.

nextid

This tag is deprecated and does not effect the document display.

xmp

listing

These tags are deprecated and not supported

## Notes

- All of the list constructs: ul,ol,menu,dir and dl support the compact attribute, which produces a more compact rendering

- If the li elements are left off of lists, the ul,/ul or pairs act as a indent operator.

- No markup may be contained within a dt element of a definition list, although any prevailing font properties are observered.

- No markup is permitted between the opening and closing title tags, otherwise the title string is truncated at the first tag.

- The application may supply its own behavior for any html tag, or override the default behavior by supplying a procedure of the form HMtag_tag or HMtag_/tag, where tag is the html tag whose behavior is to be defined. See the reference manual for details.

- The default white space and font style selection for all built-in tags may be changed by the application by changing values in either the HMinsert_map or HMtag_map. See the manual for details.

## Exceptions

- The text scrolling behavior is poor when the page contains large embedded images

- In this version, there is no direct way to change the font used in form data entry fields, you need to use the option database.

- Too much white space sometimes gets added when succesive multiple white-space separated html tags with no plain text are encountered.